##### LAKES A ZONE - SETTLEMENT MANAGEMENT AREA

#### Checklist of All District Plan Land Use Activity Rules Applicable To The Lakes A Zone –

#### Settlement Area

This checklist brings together all of the District Plan rules applicable to land use activities within the Lakes A zone - Settlement Management Area.  Its aim is to provide a comprehensive list of the rules in a useful format to assist both those proposing projects involving land use activities and Council staff assessing applications for projects.  Working through the checklist will identify:

- rules relevant to a particular project & those that are not applicable,

- the level of information required to be provided in a Project Information  Memorandum (PIM) application to ascertain compliance with rules or otherwise

- whether a land use consent is required. If so, the type of consent required.

Please note there is another checklist available which has been specifically designed for alterations and additions to existing houses and also for accessory buildings such as garages which maybe more applicable to your project.  In addition to the District Plan rules there are also regional council plans & rules that may apply to your proposal.

The Planning Services team has a Duty Planner service available free of charge to help identify which parts of the District Plan apply to your proposal.  For projects within the Lakes A Zone we strongly recommended that you or your consultant make use of this service by making an appointment with the Duty Planner as early as possible during the design of your proposal.   Please phone (07) 348 4199 for an appointment. Email enquiries can be sent to planenquiry@rdc.govt.nz.

It is hoped that by identifying relevant rules prior to designing site works and buildings the need for a land use consent may be avoided altogether.  Alternatively, if compliance can not be achieved or it is proposed to undertake activities outside of the permitted rules this can be identified early on in the planning process. The type of land use consent required can be ascertained allowing any applicable assessment criteria to be identified, considered at the time of design and addressed in any land use application.

This checklist is not intended to be a substitute for reading the Lakes A part of the District Plan.  Please consult Part Twenty of the District Plan for the complete text. Copies of the District Plan are available at the Planning Services Counter, the Rotorua Public Library, or on the Council’s website www.rdc.govt.nz under Our Services/ District Plan / Policy / District Plan.

HOW TO USE THIS CHECKLIST:

This checklist can be used EITHER by filling out a hard copy of it of it OR alternatively filling it out electronically.

Copies are available from the Planning Services Office and can be posted to you. Please phone (07) 348 4199 to arrange this.  An electronic version is available from the Rotorua District Council’s website www.rdc.govt.nz or can be emailed to you if you contact the Planning Services Dept by phoning (07) 348 4199 or by emailing planenquiry@rdc.govt.nz .

The electronic version has field codes through it where comment is required.  To turn the field codes on press Alt F9.  Press F11 to reach each field code within the checklist.

PLEASE NOTE:

  - All words within the rules shown in italic are defined in the Definitions section of the Lakes A part of the District Plan (Part Twenty).  Those most likely to be related to your project are attached as the last pages to this form.  Please consult the definitions as you come across words in italic within the checklist.

  - Numbering of the rules within this checklist are the same as that within the full text.

  - Please note that in most cases the land between the private property boundary & the existing road seal is likely to be within the zoned Sensitive Rural Management Area NOT within the Settlement Management Area. Please confirm by checking the Planning Maps.  For any work in this area such as improvement to existing access way or new works the Sensitive Rural rules need to be used.  This checklist only contains the rules applicable to land zoned Settlement.

APPLICANT’S NAME: ______________________________________________________________________________________________________

PROPERTY ADDRESS:	_____________________________________________________________________________________________________

BRIEF DESCRIPTION OF ACTIVITY: ________________________________________________________________________________________

| ###### Rule | Permitted Activity Rules | Activities requiring Land use consent | Indicate whether the proposed work: - complies or will comply with the permitted rule, (demonstrate how in application) - requires land use consent. If so what type? - or alternatively that the rule is not applicable to the activity (N/A), For Council use - whether further information is required to determine whether permitted or otherwise |
| --- | --- | --- | --- |
|  | ACTIVITIES RULES |  |  |
| 2.0 Indigenous Vegetation Disturbance | A2.1.1 Indigenous Vegetation Disturbance complying with any of the following conditions: 1. It is indigenous vegetation disturbance for the purpose of creating a building platform or vehicle access to it where: a) There is no alternative building platform clear of indigenous vegetation; and b) The indigenous vegetation is not located within the 2.5 metre buffer from the site boundary; or 2. It is disturbance of indigenous vegetation that does or will within two years obstruct the view obtained from a viewpoint; or 3. It is trimming of the indigenous vegetation, where the vegetation does or will within two years do any of the following: a) interfere with the operation of existing electricity and telecommunication lines that are Permitted Activities under Rules A37.1.1, A37.1.2, B37.1.1 and B37.1.2 or interfere with the safe operation of an existing underground gas pipeline; or b) be within 5 metres vertical height from a carriageway or the shoulder of a public road, private road, private way or an on-site manoeuvring area (i.e. overhanging branches); or c) be within 3.5 metres vertical height above or one (1) metre below the services corridor of a public road (Refer Diagram below); Note: 1. ‘A’ – Existing sealed carriageway width, or where the road is to be widened, in accordance with Rule 33.1.1. 2. All vertical measurements are to be taken from the centreline of the carriageway formation (i.e: 3.5m and 5.0m), and apply equally to each side of the road. 3. All measurements are in metres. But not where: 4. It is disturbance of indigenous vegetation associated with a geothermal feature; or 5. It is disturbance of indigenous vegetation within a RAP or a PNA; or 6. It is disturbance of indigenous mistletoe species (Tupeia Antarctica and Ileostylus micranthus) or disturbance of the host plant on which indigenous mistletoe is present; or 7. It is disturbance of planted or naturally established indigenous vegetation that is protected as a condition of a resource consent. | ## A2.2 Controlled Activities There are no Controlled Activities. ## A2.3 Restricted Discretionary Activities There are no Restricted Discretionary Activities. ## A2.4 Discretionary Activities A2.4.1 Any Indigenous Vegetation Disturbance that does not comply with the conditions for Permitted Activities. ## A2.5 Non-Complying Activities There are no Non-Complying Activities. |  |
| ### 4.0 Spec Vegtn Disturbance & Establishment Rules | 4.1 Permitted Activities 4.1.1 Exotic Tree Establishment complying with the following condition: 1. The vegetation does or will not when mature obstruct the view obtained from a viewpoint. 4.1.2 Notwithstanding any other rule, the removal of pest plants as specified in the Regional Pest Plant Management Strategy. | ## 4.2 Controlled Activities 4.2.1 Forest Harvesting on any site which is accessed from Spencer Road or Millar Road. Council shall reserve its control and may impose conditions on the following matters: * The timing of transport of exotic vegetation off-site. * Dust prevention or mitigation measures. In addition to the above, Council may impose a bond to ensure satisfaction of conditions of consent and a charge to cover monitoring costs. - Forest harvesting in the covenanted areas at Lake Tikitapu and Lake Rotokakahi as shown on DPS 54801. Council shall reserve its control and may impose conditions on the following matters: * The staging, the time of day, season and weather conditions when the harvesting can take place. * Measures to address nuisance effects such as noise. * The management of the transport of harvested logs to reduce adverse effects on the community and roading safety. * The replanting of areas of harvested forest in species and patterns that maintain the landscape values of the area and that are capable of being selectively managed. * Dust prevention of mitigation measures. ## In setting conditions on any consent, Council will consider any agreement or covenant that exists between the Crown and the Licensee that meets the objectives of this plan. In particular, Council will consider using the covenants that exist between the Crown and the Licensee dated 30 April 1990 as a basis for conditions on a consent. Because the nature of the activity, Council will consider giving a long term consent with a review clause. ## In addition to the above, Council may impose a bond to ensure satisfaction of conditions of consent and a charge to cover monitoring costs. ## 4.3 Restricted Discretionary Activities ## There are no Restricted Discretionary Activities. ## 4.4 Discretionary Activities 4.4.1 Any Exotic Tree Establishment that does not comply with the conditions for Permitted Activities. ## 4.5 Discretionary Activities There are no Non-Complying Activities. |  |
| ### 5.0 ### Earthworks | Advisory Note: - Land disturbance activities are also controlled by provisions in regional plans. 5.1 Permitted Activities A5.1.1 Earthworks complying with the following conditions: - The earthworks do not disturb any recorded heritage feature including historic places, archaeological sites and waahi tapu; and - The earthworks are on slopes that do not exceed 15o; and - The fill is cleanfill and has a vertical dimension that does not exceed 450mm; and - The excavation has a vertical dimension that does not exceed 1500mm (1.5 metres); and - The earthworks do not exceed 100m3; - The earthworks are outside an ephemeral watercourse; and - The earthworks shall not result in a change to the entry or exit point on a site of an overland flowpath, or change the catchment size of an overland flowpath, except where the earthworks are granted a consent or a permit by the regional council that specifically authorises the modification of the overland flowpath or are for the maintenance, renewal or upgrade of Rotorua Lake Council’s urban stormwater network where the discharge is authorised by a consent by the regional council; and - The earthworks are, or each planned separate stage of earthwork is, completed within 3 months. A5.1.2 Temporary stockpiling of re-useable earth or rock material complying with the following conditions: - It is outside an ephemeral watercourse, or the 2% AEP lake flood level; and - It does not exceed 100m3; and 3. It is inert; and 4. It is entirely removed within 6 months; and 5. It does not disturb any recorded heritage feature including historic places, archaeological sites and waahi tapu. | ## A5.2 Controlled Activities There are no Controlled Activities. ## A5.3 Limited Discretionary Activities A5.3.1 The earthworks are for a building platform and access to it, or for a structure and comply with the following standards: - The earthworks do not disturb any recorded heritage feature including historic places, archaeological sites and waahi tapu; and - No part of the earthworks can be seen from a lake, a viewpoint or a public reserve; and - (a) The earthworks are on slopes that exceed 15o but do not exceed 24o; or (b) The fill is cleanfill and has a vertical dimension that exceeds 450mm (0.45 metres) but does not exceed 1500mm (1.5 metres); or (c) The excavation has a vertical dimension that exceeds 1500mm (1.5 metres) but does not exceed 2.5 metres. Council shall restrict its discretion and may impose conditions in respect of the following matters: * Geotechnical design. * Land(s). * Stability management. * Any effect on an historic place, archaeological site, or waahi tapu both recorded and unrecorded. * Measures to avoid, remedy or mitigate effects on any view obtained from a lake, viewpoint, public reserve or public road. * The location on-site, the extent, method, staging, time of day, season and weather conditions under which earthworks can be undertaken. * The location of building platforms. In addition to the above, Council may impose a bond to ensure satisfaction of conditions of consent and a charge to cover monitoring costs. A5.3.2 Earthworks in ephemeral watercourses. Council shall restrict its discretion and may impose conditions in respect of the following matter: 1., Measures to avoid sedimentation from ephemeral watercourses. ## A5.4 Discretionary Activities A5.4.1 Earthworks that do not comply with the conditions for Permitted Activities or are not Restricted Discretionary Activities. ## A5.5 Non-Complying Activities There are no Non-Complying Activities. |  |
| ### 6.0 ### Building Platforms | 6.1 Permitted Activities B6.1.1 Building Platforms complying with the following conditions: - They are outside an ephemeral watercourse; and - They are at least 1 metre above the groundwater table; and - They are able to accommodate any on-site effluent treatment system; and - They can be accessed by a vehicular access that can be formed within the Permitted Activity conditions for earthworks and indigenous vegetation disturbance; and - They are located clear of any areas of instability or known natural or artificial hazard; and - They are not located within a 2.5 metre buffer of a site boundary. | ## B6.2 Controlled Activities There are no Controlled Activities. ## B6.3 Restricted Discretionary Activities B6.3.1 Any Building Platforms that do not comply with the conditions for Permitted Activities. Council shall limit its discretion and may impose conditions in respect of the matters contained in RD6.1 and RD6.2 ## B6.4 Discretionary Activities There are no Discretionary Activities. ## B6.5 Non-Complying Activities There are no Non-Complying Activities. |  |
| ### 7.0 Buildings | 7.1 Permitted Activities A7.1.1 Buildings complying with the conditions for Permitted Activities. |  |  |
| 8.0 Second Hand Buildings | 8.1 Permitted Activities A8.1.1 Resited Second Hand Buildings on sites are not Permitted Activities. | ## A8.2 Controlled Activities There are no Controlled Activities. ## A8.3 Restricted Discretionary Activities A8.3.1 Any resited second hand buildings. Council shall restrict its discretion and may impose conditions in respect of the following matters: * The external cladding and appearance of building, including assessment of whether the finish and detail is compatible with the era of the building. * The nature and timing of any upgrading work to be done to achieve a standard of appearance equivalent to a new building. * The ability to comply with the requirements of the New Zealand Building Code. * The degree to which the work facilitates the relocation of a heritage building. In addition to the above, Council may impose a bond to ensure satisfaction of conditions of consent and a charge to cover monitoring costs. ## A8.4 Discretionary Activities There are no Discretionary Activities. ## A8.5 Non-Complying Activities There are no Non-Complying Activities. |  |
| 9.0 Structures | 9.1 Permitted Activities A9.1.1 Except as provided for in Rule 37.0, structures complying with the following conditions: - Exterior surfaces including the roof, that are finished in reflectivity values between 0 and 37%; and - Except for structures accessory to a Marae, they cannot be seen from a lake or a viewpoint; and - Except for structures accessory to a Marae, they cannot be seen against the sky when viewed from a lake or public reserve); and - Except for poles for road safety lighting purposes that do not exceed 13 metres, the maximum height shall not exceed 6 metres above ground level. A9.1.2 Structures that are temporary and are located on a site or road for a period that does not exceed one (1) month in any calendar year or for the time the structure is required to complete the construction of a building. | ## A9.2 Controlled Activities There are no Controlled Activities. ## A9.3 Restricted Discretionary Activities A9.3.1 Any structures that do not comply with the conditions for Permitted Activities. Council shall restrict its discretion and may impose conditions in respect of the following matters: * Landscaping. * The height of structures. * Reflectivity values – the level of reflectivity. * The location and placement of the structure on site. * The extent to which any lighting is visible or obtrusive when seen from a lake, viewpoint or public reserve. In addition to the above, Council may impose a bond to ensure satisfaction of conditions of consent and a charge to cover monitoring costs. ## A9.4 Discretionary Activities There are no Discretionary Activities. ## A9.5 Non-Complying Activities There are no Non-Complying Activities. |  |
| 10.0 Lakes Structures | 10.1.1 The following Lake Structures: 1. Existing public boat launching ramps and adjacent associated jetties as identified on the Planning Maps and the maintenance or replacement of these boat ramps and jetties. 2. Buoys, poles, lights, beacons, and safety booms, for navigation, safety or regulatory purposes. | ## 10.2 Controlled Activities There are no Controlled Activities. ## 10.3 Restricted Discretionary Activities 10.3.1 Private lake structures where no access is available to a site from a road, private road or private way, or jetty on adjoining land in the same ownership. Council shall restrict its discretion and may impose conditions in respect of the matters contained in RD10.1 and RD 10.2. ## 10.4 Discretionary Activities 10.4.1 New public lake structures where the width does not exceed 4.2 metres, the length does not exceed 290 metres and the area does not exceed 50m². ## 10.5 Non-Complying Activities 10.5.1 New private lake structures which are not Restricted Discretionary Activities. 10.5.2 New public lake structures which are not Permitted or Discretionary Activities. . |  |

| 11.0 Hard Surfaces | A11.1.1 Hard Surfaces complying with the following condition: - Hard surface coverage not exceeding 4%. | ## A11.2 Controlled Activities There are no Controlled Activities. ## A11.3 Restricted Discretionary Activities A11.3.1 Any Hard Surfaces that do not comply with the conditions for Permitted Activities. Council shall restrict its discretion and may impose conditions in respect of the matters contained in RD11.1 and RD 11.2. ## A11.4 Discretionary Activities There are no Discretionary Activities. ## A11.5 Non-Complying Activities There are no Non-Complying Activities. |  |
| --- | --- | --- | --- |
| 12.0 Recreational Opportunities | 12.1.1 Notwithstanding any rules regulating indigenous vegetation disturbance, exotic vegetation disturbance, or earthworks, the establishment and maintenance of walking tracks to provide pedestrian access to enable the appreciation of outstanding landscapes and natural features, and historic sites complying with the following conditions - The track formation and/or vegetation disturbance is less than 1.5 metres wide; and - Recorded heritage features including historic places, archaeological sites and waahi tapu are not disturbed; c) Earth and debris is not placed in a waterbody; and d) Any walkway on Spencer Road shall be on one side of the road except for any crossing points. 12.1.2 Recreational activities that do not involve vessels or motor craft. 12.1.3 Motor craft on Lakes Rotomahana and Okataina, except for PWC (personal watercraft) and hovercraft and aircraft and helicopter landing, provided that the maximum speed shall be 5 knots within 200 metres of the shore and there shall be no person or device (other than equipment used legally for fishing) for any recreational purpose towed behind any boat. 12.1.4 Motor craft on Lakes Okareka and Tarawera, provided that the maximum speed shall be 5 knots within 200 metres of the shore, except where an access lane has been established. 12.1.5 Motor craft on Lakes Tikitapu and Okaro. 12.1.6 Existing access lanes on Lakes Okareka and Tarawera as shown on the Planning Maps. | ## 12.2 Controlled Activities There are no Controlled Activities. ## 12.3 Restricted Discretionary Activities There are no Restricted Discretionary Activities. ## 12.4 Discretionary Activities 12.4.1 New access lanes. ## 12.5 Non-Complying Activities 12.5.1 Any Recreational Activities that do not comply with the conditions for Permitted Activities. 12.5.2 Vessels on any lake. |  |

| 13.0 Heritage Features | 13.1 Permitted Activities 13.1.1 Maintenance of existing domestic gardens, buildings and structures including signs and seasonal grazing on sites or items listed in Appendix A that is part of Rule 1.1 of the Lakes A Zone (the full text version), or recorded Archaeological sites and those Maori reserves identified on the Planning Maps. - Redecoration or repair of any building listed in Appendix A that is part of Rule 1.1 of the Lakes A Zone (the full text version) to the design of and using the same or equivalent materials and appearance to those originally used. - Marae buildings on sites identified in Appendix A that is part of Rule 1.1 of the Lakes A Zone (the full text version). Provided that in regards to Rule 13.1.1 and 13.1.2 Council is to be formally advised in writing prior to any work being undertaken within a site or on an item listed in Appendix A that is part of Rule 1.1 of the Lakes A Zone (the full text version) or a recorded archaeological site. | ## 13.2 Controlled Activities There are no Controlled Activities. ## 13.3 Restricted Discretionary Activities There are no Restricted Discretionary Activities. ## 13.4 Discretionary Activities 13.4.1 Alterations, additions or reconstruction of existing buildings or structures that do not comply with the conditions for Permitted Activities. 13.4.2 Except for the placement of marae buildings, the placement of any building or structure on a site or item listed in Appendix A that is part of Rule 1.1 of the Lakes A Zone (the full text version), a recorded archaeological site, or a Maori reserve identified on the Planning Maps. 13.4.3 Excavation, construction or any other work including the use of heavy machinery immediately adjacent to or on a site or item listed in Appendix A that is part of Rule 1.1 of the Lakes A Zone (the full text version) (including within the dripline of any tree listed in this Appendix), a recorded archaeological site, or a Māori Reserve identified on the Planning Maps. Where there is doubt about the exact location and extent of the archaeological site, Council shall consider the site as officially recorded to be within a radius of 100 metres. 13.4.4 Trimming, pollarding or surgically treating a tree identified in Appendix A that is part of Rule 1.1 of the Lakes A Zone (the full text version). ## 13.5 Non-Complying Activities 13.5.1 Any Activity that does not comply with the conditions for Permitted Activities or is not a Discretionary Activity. |  |
| --- | --- | --- | --- |
| 14.0 Signs | 14.1 Permitted Activities 14.1.1 Signs complying with the following conditions: 1. They have a maximum area not exceeding 0.5m2 per site; and 2. The maximum area may be utilised by more than one sign; and 3. No signs shall be erected except as it relates to the site on which it is situated; and 4. They do not obscure a driver’s vision or cause confusion or distraction that would compromise the safe movement of traffic on a public road; and 5. They are not illuminated, flashing, animated or made of reflectorised materials. 14.1.2 Temporary signs complying with the following conditions: 1. They are not displayed for more than 6 weeks; and 2. They have a maximum area not exceeding 3m2. | ## 14.2 Controlled Activities There are no Controlled Activities. ## 14.3 Restricted Discretionary Activities There are no Restricted Discretionary Activities. ## 14.4 Discretionary Activities 14.4.1 Any signs that do not comply with the conditions for Permitted Activities. ## 14.5 Non-Complying Activities There are no Non-Complying Activities. |  |
| 15.0 Outdoor Storage | 15.1 Permitted Activities A15.1.1 Outdoor storage complying with the following conditions: - Its total volume at any one time does not exceed 10m3 unless the material is required for an on-site development or landscaping work; and - It does not generate any objectionable odour at any site boundary; and - The area on which it is stored is outside an ephemeral watercourse or the 2% AEP lake flood level; and - It is inert. | ## A15.2 Controlled Activities There are no Controlled Activities. ## A15.3 Restricted Discretionary Activities A15.3.1 Outdoor Storage that exceeds 10m3 but does not exceed 20m3 and where the material is not required for an on-site development or landscape work. Council shall restrict its discretion and may impose conditions in respect of the following matters: * Whether it generates any objectionable odour; and * Whether it can be seen from any abutting sites; and * Whether it can be seen from a lake, viewpoint, public reserve or public road; and * Whether the area on which it is stored is located within an ephemeral watercourse or the 2% AEP lake flood level. In addition to the above, Council may impose a bond to ensure satisfaction of conditions of consent and a charge to cover monitoring costs. ## A15.4 Discretionary Activities A15.4.1 Outdoor Storage where materials are to be used off-site, or processed or manufactured on-site for sale or use off-site. A15.4.2 Any Outdoor Storage that does not comply with the conditions for Permitted Activities or is not a Restricted Discretionary Activity. ## A15.5 Non-Complying Activities There are no Non-Complying Activities. |  |
| 16.0 Helicopter Landing Areas | A16.1.1 Helicopter Landing Areas are not Permitted Activities. | ## A16.3 Controlled Activities There are no Controlled Activities. ## A16.3 Restricted Discretionary Activities There are no Restricted Discretionary Activities. ## A16.4 Discretionary Activities There are no Discretionary Activities. . ## A16.5 Non-Complying Activities There are no Non-Complying Activities. ## A16.6 Prohibited Activities A16.6.1 Helicopter Landing Areas are Prohibited Activities. |  |
|  | BUILDING DESIGN RULES |  |  |
| 17.0 Site Coverage of Buildings | A17.1.1 Site coverage not exceeding 250m2 or 16 % whichever is the greater provided that on sites that exceed 2500m2 the site coverage for any single building shall not exceed 400m2. | ## A17.2 Controlled Activities There are no Controlled Activities. ## A17.3 Restricted Discretionary Activities A17.3.1 Site coverage that exceeds 250m² but does not exceed 300m² on sites that are less than or equal to 1875m². Council shall restrict its discretion to the following matters: 1. The extent to which the external design and location of the building: (a) Detracts from or promotes natural character. (b) Is visually prominent. (c) Affects the naturalness of the view from a viewpoint. 2. The extent to which the building and its principal elements are of a scale compatible with the attributes of the landscape policy area in which it is situated. 3. Where the proposal does not comply with more than one building design rule, the cumulative effect of the non-compliance. 4. The extent to which the more than minor adverse effects of the proposed building can be remedied or mitigated on-site by measures such as landscaping, retention or vegetation or re-vegetation. 5. Whether an alternative design or location on site is able to address any adverse effects. 6. The location of the building on a site in relation to other buildings and site boundaries so as not to visually link with any building on or off the site and thereby give the effect of one continuous building when viewed from a lake, or a public reserve. 7. The extent to which the building is of a scale compatible with the attributes of the landscape policy area in which it is situated. Council may impose conditions in respect of the following matters: * Specifying the: - Location on a site. - Maximum height of the building. - Maximum floor area of building(s). - Reflectivity values – The level of reflectivity. - Design themes including measures that will break the proposed building’s external form. - Separation distances from adjoining sites. - Landscaping.. - Rehabilitation of disturbed land. - Re-vegetation. - Stormwater disposal measures or off-set works. - Effluent disposal methods. - Minimum floor levels from the ground. * Require procedures including an assessment by a suitably qualified person recognised by the Historic Places Trust and/or an historic heritage site assessment or consultation with the Tangata Whenua should the construction of the building disturb a feature that has historical, cultural or spiritual significance but was not previously recorded. * Requiring measures to address nuisance effects such as noise associated with the construction of a building. * Specifying maximum glare and light levels. In addition to the above, Council may impose a bond to ensure satisfaction of conditions of consent and a charge to cover monitoring costs. ## A17.4 Discretionary Activities There are no Discretionary Activities. ## A17.5 Non-Complying Activities A17.5.1 Site coverage that does not comply with the conditions for Permitted Activities or is not a Restricted Discretionary Activity. |  |
| 19.0 Height | 19.1 Permitted Activities A19.1.1 Marae buildings where the maximum height does not exceed 7.5 metres above ground level. A19.1.2 Except for Marae buildings, any building where the maximum height does not exceed 6 metres above ground level and where the exterior walls do not exceed 5 metres in height, provided that chimneys may exceed the maximum height by up to 0.5 metres. | ## A19.2 Controlled Activities A19.2.1 Except for Marae buildings, any building where the maximum height exceeds 6 metres but does not exceed 7.5 metres and/or where the 5 metre exterior wall height is exceeded, provided that chimneys may exceed the maximum height by up to 0.5 metres. Council shall reserve its control and may impose conditions on the following matters: * Reflectivity values – The level of reflectivity. * The area of glass. * Surface treatment – the materials and finish used for the exterior. * The width of any eave. * The height of exterior walls. * Design features to break up wall or roof surface areas. * Finish on guttering. * Landscaping to reduce the visibility of a building from the lake or reserves. * Site coverage. In addition to the above, Council may impose a bond to ensure satisfaction of conditions of consent and a charge to cover monitoring costs. ## A19.3 Restricted Discretionary Activities There are no Restricted Discretionary Activities. ## A19.4 Discretionary Activities A19.4.1 Any building that does not comply with the conditions for Permitted Activities or with the standards for Controlled Activities. ## A19.5 Non-Complying Activities There are no Non-Complying Activities. |  |
| 20.0 Solar Access | 20.1 Permitted Activities A20.1.1 Any building where the height does not exceed the sunlight control plane. | ## A20.2 Controlled Activities There are no Controlled Activities. ## A20.3 Restricted Discretionary Activities A20.3.1 Any building that does not comply with the conditions for Permitted Activities. Council shall restrict its discretion to the following matters: 1. The degree of access or sunlight or the limitation of shadows on adjoining properties. 2. Whether an alternative design or location on site is able to address any adverse effects. 3. Whether the proposal does not comply with more than one design rule and the cumulative effect of the non-compliance. Council may impose conditions in respect of the following matters: * Height of buildings. * Location of building platform. * Site coverage. * Distance from boundaries. * Spacing between buildings * Height of building walls. * Confining the building dimensions within inclined solar access planes. In addition to the above, Council may impose a bond to ensure satisfaction of conditions of consent and a charge to cover monitoring costs. ## A20.4 Discretionary Activities There are no Discretionary Activities. ## A20.5 Non-Complying Activities There are no Non-Complying Activities. |  |
| 21.0 Reflectivity Values | 21.1 Permitted Activities A21.1.1 Any building where the exterior surfaces are finished, including the roof, in reflectivity values of between 0 and 37%. | ## A21.2 Controlled Activities There are no Controlled Activities. ## A21.3 Restricted Discretionary Activities There are no Restricted Discretionary Activities. ## A21.4 Discretionary Activities A21.4.1 Any building that does not comply with the conditions for Permitted Activities. ## A21.5 Non-Complying Activities There are non Non-Complying Activities. |  |
| 22.0 Viewpoints | 22.1 Permitted Activities A22.1.1 Except for Marae buildings, any building regardless of whether the building can be seen from a viewpoint, where the maximum height does not exceed 6 metres above ground level and where the exterior walls do not exceed 5 metres in height. A22.1.2 Any building that cannot be seen from a viewpoint, where the maximum height does not exceed 7.5 metres above ground level and where the exterior walls do not exceed 5 metres in height. Note: The extent of the landforms which can be seen from each viewpoint is shown in the document titled ‘Lakes A Zone – View Shaft Assessment, Rotorua District Council (March 1999)’. Reference to this document included as Appendix 15.0 of Volume 2 of the Lakes A Zone should be made to assist in the determination of whether a building is likely to be able to be seen from each viewpoint. | ## A22.2 Controlled Activities A22.2.1 Except for marae buildings, any building that can be seen from a viewpoint, where the maximum height exceeds 6 metres but does not exceed 7.5 metres and/or where the 5 metre exterior wall height is exceeded. Council shall reserve its control and may impose conditions on the following matters: * Height of buildings to reduce their visual effects on viewpoints in the Okareka and Tarawera catchments. * Reflectivity values – the level of reflectivity. * The area of glass. * Surface treatment – the materials and finish used for the exterior. * The width of any eave. * The height of exterior walls. * Design features to break up wall or roof surface areas. * Finish on guttering. Landscaping to reduce the visibility of a building from the lake or reserves. In addition to the above, Council may impose a bond to ensure satisfaction of conditions of consent and a charge to cover monitoring costs. ## A22.3 Restricted Discretionary Activities A22.3.1 There are no Restricted Discretionary Activities. ## A22.4 Discretionary Activities A22.4.1 Any building that does not comply with the conditions for Permitted Activities or with the standards for Controlled Activities. ## A22.5 Non-Complying Activities There are no Non-Complying Activities. |  |
| 23.0 Skylines | 23.1 Permitted Activities A23.1.1 Except for Marae buildings, any building that cannot be seen against the sky when viewed from a lake or public reserve. | ## A23.2 Controlled Activities There are no Controlled Activities. ## A23.3 Restricted Discretionary Activities There are no Restricted Discretionary Activities. ## A23.4 Discretionary Activities A23.4.1 Any building that does not comply with the conditions for Permitted Activities. ## A23.5 Non-Complying Activities There are no Non-Complying Activities. |  |
| 24.0 Floor Levels | 24.1 Permitted Activities A24.1.1 Any habitable building where the floor level is at least 1.5 metres above the groundwater table. | ## A24.2 Controlled Activities There are no Controlled Activities. ## A24.3 Restricted Discretionary Activities A24.4.1 Any building that does not comply with the conditions for Permitted Activities. Council shall restrict its discretion and may impose conditions in respect of the following matters: * Whether the proposal does not comply with more than one building design rule and the cumulative effect of the non-compliance. * Whether an alternative design or location on site is able to address any adverse effects. * Specifying minimum floor levels. In addition to the above, Council may impose a bond to ensure satisfaction of conditions of consent and a change to cover monitoring costs. ## A24.4 Discretionary Activities There are no Discretionary Activities.. ## A24.5 Non-Complying Activities There are no Non-Complying Activities. |  |
| 25.0 Buffers | 25.1 Permitted Activities A25.1.1 Any building not located within 2.5 metres of a site boundary. | ## A25.2 Controlled Activities There are no Controlled Activities. ## A25.3 Restricted Discretionary Activities There are no Restricted Discretionary Activities. ## A25.4 Discretionary Activities A25.4.1 Any building that does not comply with the conditions for Permitted Activities. ## A25.5 Non-Complying Activities There are no Non-Complying Activities. |  |
|  | EFFECTS RULES |  |  |
| 26.0 Electromagnetic Radiation | 26.1 Permitted Activities 26.1.1 Any activity producing radio frequency energy shall meet the recommended non-occupational limits set out in the New Zealand Standard NZS 2772.1 (1999) – Maximum Exposure Levels – 3kHz to 300 GHz at any point where any member of the public may be exposed to the source of such energy. | ## A26.2 Controlled Activities There are no Controlled Activities. ## A26.3 Restricted Discretionary Activities There are no Restricted Discretionary Activities. ## A26.4 Discretionary Activities There are no Discretionary Activities. . ## A26.5 Non-Complying Activities A26.5.1 Any activity that does not comply with the conditions for Permitted Activities. |  |
| 27.0 Spill Light or Stray Light Emissions | 27.1 Permitted Activities A27.1.1 Any activity that results in spill light or stray light emissions complying with the following condition: 1. Any direct artificial illumination shall not exceed ten (10) lux when measured at the nearest site boundary. Illuminance levels shall be measured vertically and horizontally in accordance with professional illumination engineering practice. A27.1.2 Any activity that results in glare complying with the following condition: 1. Lighting shall be aimed no higher than 30o below the horizontal and shall be aimed, hooded or screened, to minimise glare. | ## A27.2 Controlled Activities There are no Controlled Activities. ## A27.3 Restricted Discretionary Activities There are no Restricted Discretionary Activities. ## A27.4 Discretionary Activities There are no Discretionary Activities. . ## A27.5 Non-Complying Activities A27.5.1 Any activity that does not comply with the conditions for Permitted Activities. |  |

| 28.0 Noise | 28.1 Permitted Activities 28.1.1 Activities complying with the following condition: 1. The provisions of Appendix 5 of Volume 2 of the Lakes A Zone. | ## 28.2 Controlled Activities 28.2.1 The noise generated at the time of the forest harvesting that exceeds the levels specified in Appendix 5 of Volume 2, of the Lakes A Zone, provided that the machinery used has mufflers and silencers installed and is operated in accordance with the manufacturers’ specifications. Council shall reserve its control and may impose conditions on the following matters: * Staging, the time of day, season and weather conditions when specified aspects of harvesting can take place. * Specifying the on-site location and design of log handling and static noise areas in relation to site boundaries. In addition to the above, Council may impose a bond to ensure satisfaction of conditions of consent and a charge to cover monitoring costs. ## 28.3 Restricted Discretionary Activities There are no Restricted Discretionary Activities. ## 28.4 Discretionary Activities 28.4.1 Any Activity that does not comply with the conditions for Permitted Activities or is not a Controlled Activity. ## 28.5 Non-Complying Activities There are no Non-Complying Activities. |  |
| --- | --- | --- | --- |
| 29.0 Traffic Generation | 29.1 Permitted Activities A29.1.1 Traffic generation complying with any of the following conditions: - It is generated from up to two houses per site; or - It is generated from a Marae; or - It is generated from public reserves; or - The number of vehicle movements does not exceed 12 light vehicle movements per day from any site, averaged over a week provided that, for the purpose of assessing compliance with this condition, when an activity occurs on a site on which house(s) are also located, each house shall be deemed to generate 6 light vehicle movements per day. | ## A29.2 Controlled Activities There are no Controlled Activities. ## A29.3 Restricted Discretionary Activities A29.3.1 Traffic generation from three houses per site of where traffic generation exceeds 12 but does not exceed 18 light vehicle movements per day from any site, averaged over a week. Council shall restrict its discretion and may impose conditions in respect of the following matters: In addition to the above, Council may impose a bond to ensure satisfaction of conditions of consent and a charge to cover monitoring costs. * The width and shape of accessways and the adjoining road. * Sight distances in either direction. * The volume and timing of traffic flows. * Whether a State Highway is involved. * The safety of pedestrians and cyclists. * Whether on-road carparking is generated. * The impact on the capacity of the adjacent roading network caused by the expected traffic generation of the proposal. * Whether there is a need to upgrade the adjacent roading network. For the purpose of assessing compliance with this Rule, when an activity occurs on a site on which house(s) are also located, each house shall be deemed to generate 6 light vehicle movements per day. In addition to the above, Council may impose a bond to ensure satisfaction of conditions of consent and a charge to cover monitoring costs. ## A29.4 Discretionary Activities A29.4.1 Traffic generation that does not comply with the conditions for a Permitted Activity or is not a Restricted Discretionary Activity. For the purpose of assessing compliance with this Rule, when an activity occurs on a site on which houses) are also located, each house shall be deemed to generate 6 light vehicle movements per day. ## A29.5 Non-Complying Activities There are no Non-Complying Activities. |  |
|  | INFRASTRUCTURE AND UTILITY RULES |  |  |
| 30.0 On-site Carparking and Turning | 30.1 Permitted Activities 30.1.1 Any activity complying with the following condition: 1. The on-site carparking and manoeuvring provisions of Appendix F of the Rule 1.1 of the Lakes A Zone (the full text version). | ## 30.2 Controlled Activities There are no Controlled Activities. ## 30.3 Restricted Discretionary Activities There are no Restricted Discretionary Activities. ## 30.4 Discretionary Activities 30.4.1 Any Activity that does not comply with the conditions for Permitted Activities. ## 30.5 Non-Complying Activities There are no Non-Complying Activities. |  |
| 31.0 Vehicle Crossings | 31.1 Permitted Activities 31.1.1 Vehicle Crossings – Light Vehicles Any activity involving light vehicle movements to or from any road complying with the following condition unless Rules 31.1.4 or 31.1.5 apply: 1. There shall be a vehicle crossing provided to each site that is formed, drained, constructed and sealed in accordance with Plan No. RD22 (Refer Appendix 3.0 of Volume 2 of the Lakes A Zone). 31.1.2 Vehicle Crossings – Commercial Vehicles Any activity involving commercial vehicle movements to or from a low volume or low speed sealed road complying with the following condition: 1. There shall be a vehicle crossing provided to each site that is formed, drained, constructed and sealed in accordance with Plan No. RD23. (Refer Appendix 3.0 of Volume 2 of the Lakes A Zone). 31.1.3 Vehicle Crossings – Commercial Vehicles Any activity involving commercial vehicle movements to or from a high volume or high speed sealed road complying with the following condition: 1. There shall be a vehicle crossing provided to each site that is formed, drained, constructed and sealed in accordance with Plan No. RD24. (Refer Appendix 3.0 of Volume 2 of the Lakes A Zone). 31.1.4 Vehicle Crossings – Unsealed Local Road Any activity involving any vehicle movements to or from an unsealed local road complying with the following conditions: 31.1.5 1. There shall be a metalled vehicle crossing provided to each site that is otherwise formed, drained and constructed in accordance with the requirements for sealed crossings. 31.1.6 Vehicle Crossings – Any Activity From a State Highway Notwithstanding the above, any activity involving any vehicle movement to or from a State Highway complying with the following condition: 1. There shall be a metalled vehicle crossing provided to each site that is formed, drained and constructed and sealed in accordance with the standards of the Road Controlling Authority. | ## 31.2 Controlled Activities There are no Controlled Activities. ## 31.3 Restricted Discretionary Activities 31.3.1 Except for vehicle crossings on State Highways, any activity that does not comply with the vehicle crossing conditions for Permitted Activities Council shall restrict its discretion and may impose conditions in respect of the following matters: * The width and shape of accessways and the adjoining road. * Sight distances in either direction. * The nature and characteristics of traffic generated and the timing of traffic flows. * The safety of pedestrians or cyclists. In addition to the above, Council may impose a bond to ensure satisfaction of conditions of consent and a charge to cover monitoring costs. ## 31.4 Discretionary Activities 31.4.1 Any activity from a State Highway that does not comply with the conditions for Permitted Activities. ## 31.5 Non-Complying Activities There are no Non-Complying Activities. |  |

| 32.0 Private Roads and Private Ways | 32.1 Permitted Activities 32.1.1 Any activity complying with the following condition: 1. The requirements for Private Roads or Private Ways for Permitted Activities as set out in Appendix 10.0 of Volume 2 of the Lakes A Zone. | ## 32.2 Controlled Activities There are no Controlled Activities. ## 32.3 Restricted Discretionary Activities 32.3.1 Private Roads or Private Ways that do not comply with the conditions for Permitted Activities, but comply with the standards for Restricted Discretionary Activities set out in Appendix 10.0 of Volume 2 of the Lakes A Zone. Council shall restrict its discretion and may impose conditions in respect of the following matters: * Topography of the land. * The nature and characteristics of traffic generated and the timing of traffic flows. * Safety of pedestrians and cyclists. ## 32.4 Discretionary Activities 32.4.1 Any Activity that does not comply with the standards for Restricted Discretionary Activities set out in Appendix 10.0 of Volume 2 of the Lakes A Zone. 32.4.2 Any Activity that complies with the standards for Discretionary Activities set out in Appendix 10.0 of Volume 2 of the Lakes A Zone. ## 32.5 Non-Complying Activities There are no Non-Complying Activities. |  |
| --- | --- | --- | --- |
| 33.0 Roads | 33.1 Permitted Activities 33.1.1 Maintenance and upgrading of the existing formation of existing roads complying with the following condition: 1. Notwithstanding the vehicle crossing Rule 31.1.5 and notwithstanding the hard surface rule (Rule 11.0), of this Volume (Volume 1), the hard surface coverage on public roads not exceeding 350mm per metre width of road. Note: A 20 metre road reserve may have a 7 metre formed sealed carriageway and shoulder. Note: If it is intended to transfer a private road to a Road Controlling Authority, the system must comply with the conditions of Asset Transfer of the Road Controlling Authority concerned. | ## 33.2 Controlled Activities There are no Controlled Activities. ## 33.3 Restricted Discretionary Activities 33.3.1 Construction of new road formations or realignment of existing road formations. Council shall restrict its discretion and may impose conditions in respect of the following matters: * Whether the hard surface coverage on public roads, excluding intersections, exceeds 350mm per metre width of road; and * Whether they are designed and constructed with a design life of a minimum of 25 years, except for vehicle paving surfaces which may be designed for resurface treatment in 12 year stages; and * Whether they are designed and constructed in accordance with Appendix 2.0 Criteria and Standards for Public Roads (Refer Volume 2 of the Lakes A Zone); and * Whether stormwater can be managed without increasing flows into a lake. In addition to the above, Council may impose a bond to ensure satisfaction of conditions of consent and a charge to cover monitoring costs. Note: If it is intended to transfer a private road to a Road Controlling Authority, the system must comply with the conditions of Asset Transfer of the Road Controlling Authority concerned. ## 33.4 Discretionary Activities 33.4.1 Maintenance of existing road formations or the construction of new road formations that does not comply with the conditions for Permitted Activities or the standards for Restricted Discretionary Activities. 33.4.2 Buildings and structures on roads. ## 33.5 Non-Complying Activities There are no Non-Complying Activities. |  |
| 34.0 Potable Water Supply | 34.1 Permitted Activities 34.1.1 Water supply systems complying with the following conditions: - Every habitable building shall be provided with a potable water supply system and either: a) At the time of applying for a Building Consent, evidence shall be supplied that the on-site potable water supply complies with section 14(3)(a) and (b) of the RMA; or b) There shall be a connection to an operational private or public community water supply system that meets the conditions for a Permitted Activity. - Every habitable building shall be provided with a water supply adequate for firefighting purposes in accordance with the New Zealand Fire Service Firefighting Water Supplies Code of Practice SNZ PAS 4509: 2008. 34.1.2 Operational private or public community water supply systems complying with the following conditions: - The system is designed and constructed for a functional life of a minimum of 50 years; and - The system is constructed to prevent leakage of water and easy connection for service connection fittings; and - Each site has practicable access to enable connection to the system; and - The system delivers to every habitable building a minimum of 20 metre head and 30 litres per minute. Note: If it is intended to transfer a private community water supply system to a Utility Operator, the system must comply with the conditions of Asset Transfer of the Utility Operator concerned. | ## 34.2 Controlled Activities There are no Controlled Activities. ## 34.3 Restricted Discretionary Activities There are no Restricted Discretionary Activities. ## 34.4 Discretionary Activities 34.4.1 Any Activity that does not comply with the conditions for Permitted Activities. ## 34.5 Non-Complying Activities There are no Non-Complying Activities. Note: An applicant is advised that non-compliance with regional rules or where a water permit has not been obtained for a private or public community water supply system, will need to be addressed with Environment BOP. |  |

| 35.0 Collection and Disposal of Stormwater | 35.1 Permitted Activities 35.1.1 All buildings and all hard surfaces (including all driveways), shall be provided with a stormwater collection and disposal system within the site complying with the following conditions: - It is designed to accommodate at least a 10% AEP storm event by: - Soakage; - Vegetated swales; - Ponding; - Wetlands; and - Surface water does not enter habitable buildings from a 2% AEP storm event; and - Where the system uses pipes that: - accessible inspection chambers are provided at all changes of grade and direction; and - self cleansing velocities are maintained; and - the system has a functional design life of 50 year. Note: If it is intended to transfer a private stormwater collection and disposal system to a Utility Operator, the system must comply with the conditions of Asset Transfer of the Utility Operator concerned. | ## 35.2 Controlled Activities There are no Controlled Activities. ## 35.3 Restricted Discretionary Activities 35.3.1 Any system intended for the collection and disposal of stormwater from buildings and hard surfaces not within the site. Council shall restrict its discretion and may impose conditions in respect of the following matters: * Whether the system is designed to accommodate at least a 10% AEP storm event by: (a) Soakage; (b) Vegetated swales; (c) Ponding; (d) Wetlands; and * Whether surface water from a 2% AEP storm event will enter any habitable buildings; and * Where the system uses pipes that: (a) Accessible inspection chambers are provided at all changes of grade and direction; and (b) Self cleansing velocities are maintained; and (c) The system has a functional design life of 50 years. * Any effect on land stability or from stormwater flows on neighbouring properties. In addition to the above, Council may impose a bond to ensure satisfaction of conditions of consent and a charge to cover monitoring costs. Note: An applicant is advised that non-compliance with regional rules or where a permit has not been obtained for a stormwater discharge will need to be addressed with Environment Bay of Plenty. ## 35.4 Discretionary Activities There are no Discretionary Activities. ## 35.5 Non-Complying Activities 35.5.1 Any Activity that does not comply with the conditions for Permitted Activities or is not a Restricted Discretionary Activity. |  |  |
| --- | --- | --- | --- | --- |
| 36.0 Sewage Collection and Disposal | 36.1 Permitted Activities 36.1.1 Sewage collection and disposal systems complying with the following conditions: - Every habitable building shall be provided with a sewage collection and disposal system and either: - There shall be an on-site sewage collection and disposal system (that may include a separate grey water disposal system) that complies with the regional rules or a regional consent is held; or - There shall be a connection to an operational community sewage collection and disposal system that meets the conditions for a Permitted Activity. - Operational private or public community sewage disposal systems designed to cater for minimum flows of 220 litres per head per day and a peak factor of four complying with the following conditions: - Each site has practicable access to enable connection to the system; and - The system is designed and constructed to eliminate the ingress of stormwater and ground water and also avoids the occurrence of system surcharging or overflows; and - The reticulation system is designed so that each site is provided with a minimum 100mm diameter connection to a minimum of 500mm inside the site boundary and at a depth capable of servicing all habitable buildings on the site; and - Accessible inspection chambers are provided at all changes of grade, direction and pipe size; and - That self-cleansing velocities are maintained within reticulation systems; and - The reticulation and pumping system be designed and constructed to allow the passing of 75mm solids; and - The reticulation and disposal system is designed and constructed for a minimum functional life of 50 years, except for electrical and mechanical equipment which may be designed and constructed for a functional life of 15 years. Note: If it is intended to transfer a private sewage collection and disposal system to a Utility Operator, the system must comply with the conditions of Asset Transfer of the Utility Operator concerned. | ## 36.2 Controlled Activities There are no Controlled Activities. ## 36.3 Restricted Discretionary Activities 36.3.1 Private sewage pumping stations. Council shall restrict its discretion and may impose conditions in respect of the following matters: * Whether private sewage reticulation systems are not able to discharge via gravity. * Whether ownership and responsibility of the private sewage pumping station relating to the design, maintenance and operational procedures is to be registered on the land title. In addition to the above, Council may impose a bond to ensure satisfaction of conditions of consent and a charge to cover monitoring costs. ## 36.4 Discretionary Activities 36.4.1 Activities that do not comply with the conditions for Permitted Activities or the standards for Restricted Discretionary Activities. Note: An applicant is advised that non-compliance with regional rules or where a permit has not been obtained for a discharge from a private or public community sewage disposal system, will need to be addressed with Environment Bay of Plenty. ## 36.5 Non-Complying Activities There are no Non-Complying Activities. |  |  |
| 37.0 Electricity and Telecommunication Lines | 37.1 Permitted Activities A37.1.1 Above ground utilities complying with the following condition: 1. Existing electric lines, including support structures for these lines, and lines defined by Section 2(1) and (1A) of the Telecommunications Act 1987, where the lines are above ground and the activity involves the operation, maintenance, replacement, upgrading (including increases in voltage and/or capacity), resiting or repositioning of existing above ground services (including their foundations), within a three dimensional corridor over the entire length of a line existing as at the time this Plan was Publicly Notified (13 May 2000) consisting of the following dimensions: - Corridor height: the greater of either 10 metres above ground level or the height of the existing support structures including conductors; and - Corridor width: the maximum width of the existing support structures including conductors. The height in (a) above may be exceeded by up to 4 metres for earth wires, earth peaks and lightning rods. A37.1.2 Below ground electricity and telecommunication lines complying with the conditions for Permitted Activities in Rules 2.0, 3.0 and 5.0. | ## A37.2 Controlled Activities There are no Controlled Activities. ## A37.3 Restricted Discretionary Activities A37.3.1 Existing above ground electric lines, including support structures, and existing above ground lines defined by Section 2(1) and (1A) of the Telecommunications Act 1987 where the: (a) Corridor height exceeds either 10 metres or the height of the existing support structures including conductors; and (b) Complies with the corridor width for Permitted Activities. Council shall restrict its discretion and may impose conditions in respect of the following matter: - Whether the visual effects on a viewpoint can be avoided, remedied or mitigated. In addition to the above, Council may impose a bond to ensure satisfaction of conditions of consent and a charge to cover monitoring costs. ## A37.4 Discretionary Activities A37.4.1 Activities that do not comply with the conditions for Permitted Activities or is not a Restricted Discretionary Activity. A37.4.2 Activities that do not comply with the conditions for Permitted Activities in Rules 2.0, 3.0 and 5.0 ## A37.5 Non-Complying Activities There are no Non-Complying Activities. |  |  |
| 38.0 Introduction to Subdivision (Subdivision Rules For The Management Areas) | 38.1 Permitted Activities A38.1.1 Subdivision is not a Permitted Activity. | ## B38.2 Controlled Activities There are no Controlled Activities. ## B38.3 Restricted Discretionary Activities A38.3.1 Subdivision complying with the following standards: - Each site shall have a minimum area of at least 2000m2; and - Each site shall contain a building platform of at least 250 m2 that complies with the conditions for building platforms as a Permitted Activity (Refer Rule 6.0); and - Each site shall contain a building platform that would not necessitate the disturbance of indigenous vegetation or a land use consent required for any disturbance of indigenous vegetation shall have been obtained; and - Each site shall at the time of subdivision comply with the conditions for hard surface coverage as a Permitted Activity; and - Each site shall be of such dimensions so as to include a circle with a diameter of 20 metres that incorporates the building platform and is inclusive of buffers; and - The Standards of Rule 40.3. Council shall restrict its discretion and may impose conditions in respect of the following matters: - Specifying the size of any site to accommodate a complying building platform. - The location of building platforms away from skylines, ridgelines and viewpoints. - Limiting disturbance of vegetation. - The shape of the site. - The extent to which natural hazard risks are avoided or mitigated and whether the activity would worsen any natural hazard. ## B38.4 Discretionary Activities B38.4.1 Subdivision that does not comply with the standards for Restricted Discretionary Activities. ## B38.5 Non-Complying Activities B38.5.1 Subdivision that does not comply with the standards for Discretionary Activities. |  |  |
| 39.0 Subdivision Rules For Roads, Private Roads and Private Ways, Reserves, Utility Services and Boundary Adjustments | 39.1 Permitted Activities 39.1.1 Subdivision is not a Permitted Activity. | ## 39.2 Controlled Activities There are no Controlled Activities. ## 39.3 Restricted Discretionary Activities Notwithstanding any other subdivision rule: 39.3.1 Subdivision for Roads Subdivision to create sites for roads complying with the following standards: 1. The hard surface coverage shall comply with the Permitted Activity conditions for roads; and 2. The matters specified in Appendix 2.0 of Volume 2 of the Lakes A Zone. Council shall restrict its discretion and may impose conditions in respect of the following matters: - Vesting of the land in the appropriate Roading Authority. - The standard of formation in relation to the function of the road and traffic characteristics in accordance with the standards specified in Appendix 2.0 of Volume 2 of the Lakes A Zone. 39.3.2 Subdivision for Private Roads and Private Ways Subdivision to create sites for private roads and private ways complying with the following standards: 1. The matters specified in Appendix 10.0 of Volume 2 of the Lakes A Zone. Council shall restrict its discretion and may impose conditions in respect of the following matter: - The standard of formation in relation to the function of the private road and private ways in accordance with the standard specified in Appendix 10.0 of Volume 2 of the Lakes A Zone. 39.3.3 Subdivision for Reserves Subdivision to create sites for reserves complying with the following standard: 1. No buildings shall be erected. Council shall restrict its discretion and may impose conditions in respect of the following matters: - Nominating the purpose of the reserve. - Vesting of the reserve in the appropriate management authority. 39.3.4 Subdivision for Boundary Adjustments Boundary Adjustments between existing sites complying with the following standards: 1. The sites shall be abutting; and 2. There shall be no additional Certificates of Title created before and after the subdivision; and 3. Any site created shall comply with the standards for the relevant Management Area or the degree of non-compliance with those standards shall not be increased before and after the subdivision; and 4. Each site created after the boundary adjustment shall be able to meet the conditions for building platforms as Permitted Activities (refer Rule 6.0); and 5. Each site created after the boundary adjustment shall be able to meet the site coverage requirements for the relevant management area (refer Rule 17.0) or the degree of non-compliance with those rules shall not be increased; and 6. The Standards of Rule 40.3. (a) The stopping or closing or a road; and (b) The creation of a separate certificate of title under the Public Works Act 1981, or under the Local Government Act 1974, or by way of survey office plan, or under any previous enactment for land severance purposes and where such sites have an area of less than 5 hectares and are not being used for a public work or network utility; and (c) Where there is no building platform that could comply with the conditions for building platforms as a Permitted Activity (Refer Rule 6.0). Council shall restrict its discretion and may impose conditions in respect of the following maters: * Limits on the area of land involved in the boundary adjustment to ensure: - Boundaries are in practical locations. - Site coverage requirements can be met. - The degree of non-compliance is not increased. * Complying building platforms ensured and where necessary nominated. * Complying building platforms ensured and where necessary nominated. 39.3.5 Subdivision for Network Utilities Subdivision to create sites for network utilities complying with the following standards: 1. Each site shall contain a building platform that: (a) Complies with the conditions for building platforms as a Permitted Activity (refer Rule 6.0); and (b) Can be located so that buildings cannot be seen from a viewpoint; and (c) Would not necessitate the disturbance of indigenous vegetation or that a land use consent required for any disturbance of indigenous vegetation shall have been obtained. 2. The General Subdivision Standards in Rule 40.3.1(1-6). Council shall restrict its discretion and may impose conditions in respect of the following matters: * The size and shape of the site. * The location of building platforms away from skylines, ridgelines and viewpoints. * The size and placement of buildings. * Controlling the disturbance of indigenous vegetation. * Controlling earthworks. * Where required for a particular network utility, the provision of any relevant services as listed in the General Subdivision Standards in Rule 40.3.1(1-12). ## 39.4 Discretionary Activities 39.4.1 Subdivision to create sites for roads, private roads and private ways, network utilities or reserves that do not comply with the standards for Restricted Discretionary Activities. ## 39.5 Non-Complying Activities 39.5.1 Subdivision for Boundary Adjustments that does not comply with the standards for Restricted Discretionary Activities. |  |  |
| 40.0 General Subdivision Standards | ## 40.3 General Standards for Subdivision as a Restricted Discretionary or Discretionary Activity 40.3.1 In addition to all preceding subdivision rules the following standards and terms shall apply to all subdivisions in all Management Areas in the Lakes A Zone. Standards applicable to all subdivision: - On-Site Access It shall be demonstrated that access can be provided to land within any nominated building platform clear of: (a) Any riparian area. (b) Any ephemeral watercourse. (c) Any land within the 2% AEP Lake Flood Level. (d) Any land with slopes exceeding 24°. - Site Boundaries The boundaries of any site shall follow landform features to enable practical land management. - Management Area Boundaries The boundary of any site forming the boundary between a Settlement Management Area and any Sensitive Rural Management Area shall be placed so as to allow reasonable vehicular access along the base or top of any hill face steeper than 15° in the non-settlement area, unless the hill face is, or is to be, a reserve. - Earthworks Any land use consent required for any earthworks to be carried out as part of the subdivision shall have been obtained. In the case of Restricted Discretionary Activities, Council shall restrict its discretion and impose conditions in respect of the following matters: * Location and size of building platforms. * Location of site boundaries. * The location on-site, the extent, staging, time of day, season and weather conditions under which earthworks can be undertaken. - Bond A bond may be imposed to ensure satisfaction of any condition(s) of consent. - Monitoring Conditions may be imposed to: (a) Monitor the performance of any condition of consent; or (b) To impose a specific charge to cover monitoring costs. - Sewage Disposal For each site containing a building platform; either: (a) It shall be demonstrated that each site is capable of supporting an on-site effluent disposal system that meets the sewage disposal Permitted Activity conditions for such systems; or (b) Each site shall be provided with a connection to an operational private or public community sewage disposal system that meets the sewage disposal Permitted Activity conditions for such systems; and In the case of Restricted Discretion Activities, Council shall restrict its discretion and impose conditions in respect of the following matter: * The Matters set out in Rule 36.1.1.2. - Water Supply For each site containing a building platform; either (a) It shall be demonstrated that each site has available a potable water supply from a nominated source; and (b) Where the nominated supply is an operational private or public community water supply system, and that system complies with the conditions for a Permitted Activity, there shall be a connection to the system provided at the boundary of each site. In the case of Restricted Discretion Activities, Council shall restrict its discretion and impose conditions in respect of the following matter: * The Matters set out in Rule 34.1.1. and 34.1.2. - Stormwater The design of the subdivision shall make provision for the anticipated requirements for the collection and disposal of stormwater for each site, and access to it, to accommodate any building or hard surface that would be permitted. In the case of Restricted Discretionary Activities, Council shall restrict its discretion and impose conditions in respect of the following matter: * The Matters set out in Rule 35.1.1. - Access and Vehicle Crossings Each site shall be provided with access to a road or private road or private way. Where the location of an access is either confined by site boundaries or limited for traffic safety reasons, a vehicle crossing shall be provided in accordance with the Permitted Activity rules for vehicle crossings. In the case of Restricted Discretionary Activities, Council shall restrict its discretion and impose conditions in respect of the following matter: * The Matters set out in Rule 31.1. - Private Roads and Private Ways Any private road or private way shall be provided in accordance with the relevant rules for Private roads and Private Ways. In the case of Restricted Discretionary Activities, Council shall restrict its discretion and impose conditions in respect of the following matter: * The Matters set out in Appendix 10.0 of Volume 2 of the Lakes A Zone. - Roads Any road shall be provided in accordance with the relevant rules for roads and shall be provided with a street naming sign. In the case of Restricted Discretionary Activities, Council shall restrict its discretion and impose conditions in respect of the following matter. * The Matters set out in Rule 33.1.1 and Appendix 2.0 of Volume 2 of the Lakes A Zone. - Esplanade Reserves (a) Where an esplanade reserve or strip is to be taken adjacent to a lake with a controlled level, the esplanade instrument shall be measured from the edge of the water when the lake is at its maximum permitted operating level. It shall not be assumed that lakes are level (flat), and the actual edge of the water when the lake is as its maximum operating level shall be determined by survey. Note: The maximum operating level (Moturiki Datum) of Lake Okareka at its outlet (set in the year 2000) is 353.90 metres. (b) On the creation of sites under 4 ha adjoining: * Lakes over 8 ha; or * Rivers over 3 metres within or adjacent to the Tarawera and Okareka Settlement or Bush Settlement Areas; or * The Wairoa River where it is over 3 metres wide: 20 metre esplanade reserves shall be required on all margins. Any esplanade strip already present may be converted to an esplanade reserve in conformity with this rule. In the case of Restricted Discretionary Activities, Council shall restrict its discretion and impose conditions in respect of the following matter: * Where an esplanade reserve is required that exceeds 20 metres, regard shall be had to Policy P15.2 and matters relating to compensation. (c) On the creation of sites under 4 ha adjoining rivers over 3 metres wide, except as identified in Rule 40.3.1.13(b), esplanade strips not less than 5 metres wide shall be created on both sites. Where an esplanade reserve strip exceeds 5 metres, the widths shall be determined by: * Natural Processes * Existing indigenous vegetation * Topography * Buffering function * Practicality * Access requirements * Indigenous fauna present In the case of Restricted Discretionary Activities, Council shall restrict its discretion and impose conditions in respect of the following matter: * Where an esplanade reserve is required that exceeds 20 metres, regard shall be had to Policy P15.2 and matters relating to compensation. (d) On the creation of sites under 4 ha adjoining permanently flowing rivers less than 3 metres wide within the Tarawera and Okareka Settlement and Bush Settlement Areas, reserves of not less than 5 metres wide shall be created on both sides. Where a reserve exceeds 5 metres, the width shall be determined by: * Natural Processes * Existing indigenous vegetation * Topography * Buffering function * Practicality * Access requirements * Indigenous fauna present In the case of Restricted Discretionary Activities, Council shall restrict its discretion and impose conditions in respect of the following matter: * Where an esplanade reserve is required that exceeds 20 metres, regard shall be had to Policy P15.2 and matters relating to compensation. - Land Improvement Agreements (e) Where a LIA (Land Improvement Agreement) already exists in situations identified in Rule 40.3.1.13(b), esplanade reserves will be required in accordance with that rule; and * Where the area retired under the LIA (Land Improvement Agreement) is less than 20 metres wide the esplanade reserve required will be 20 metres; * Where the area retired is more than 20 metres wide the esplanade reserve will: - Have a similar width unless there are exceptional circumstances, but may exclude areas identified within the LIA (Land Improvement Agreement) as being available for productive uses; or - Be 20 metres wide with the balance protected by an alternative protective instrument. In the case of Restricted Discretionary Activities, Council shall restrict its discretion and impose conditions in respect of the following matter: * Where an esplanade reserve is required that exceeds 20 metres, regard shall be had to Policy P15.2 and matters relating to compensation. ## 40.4 Discretionary Activities 40.4.1 Subdivision that does not comply with the General Standards for Subdivision 2, 7, 8, 9, 10, 11, 12 and 14 as Restricted Discretionary Activities. ## 40.5 Non-Complying Activities 40.5.1 Subdivision that does not comply with the General Standards for Subdivision 1, 3, 4, and 13 as Restricted Discretionary Activities. |  |  |  |
| ## 41.0 ## Consultation with Tangata Whenua | ## 41.2 General Standards for Land Use and Subdivision as a Controlled or Restricted Discretionary Activity 41.2.1 In addition to all preceding rules, the following standards and terms shall apply to all Controlled and Restricted Discretionary Activity applications and applies in All Management Areas within the Lakes A Zone. Standards applicable to all Controlled and Restricted Discretionary Activities. - At the time of lodgement of any application for Resource Consent, the application shall be referred to nominated Tangata Whenua representative of the hapu holding Mana Whenua status for the site that is the subject of the application. ## 41.3 General Standards for Land Use and Subdivision as a Discretionary or Non-Complying Activity 41.3.1 In addition to all preceding rules, the following standards and terms shall apply to all Resource Consent applications, and applies in All Management Areas within the Lakes A Zone. Standards applicable to all Discretionary and Non-Complying Activities. - Prior to the lodgement of any application for Resource Consent, consultation shall be required with nominated Tangata Whenua representatives of the hapu holding Mana Whenua status for the site that is the subject of the application. |  |  |  |
| ## 42.0 Schedules A42.0 Schedule 1 Lot 2 DPs 12358 Ronald Road Anticipated Environmental Results 1. Operation and maintenance of exiting tourist lodge. 2. Maintenance and enhancement of the character and amenity of the surrounding settlement. 3. Protection and enhancement of the natural values associated with Lake Tarawera and its margins. 4. Minor changes of scale and intensity of activity on the site. 5. Minor adverse effects on the environment. | ## 42.1 Permitted Activities A42.1.1 There are no Permitted Activities. |  | ## A42.2 Controlled Activities A42.2.1 The following buildings and activities are Controlled Activities: 1. A maximum of eleven guest bedrooms, including occasional use of the manager’s accommodation for guest accommodation; 2. Manager’s accommodation. 3. Communal dining and living facilities; 4. Buildings and activities accessory to the foregoing Controlled Activities; and complying with the following standards. Buffers: 2.5 metres. Height: 7.5 metres. Site coverage not exceeding 25%. Hard surfaces not exceeding 15%. Parking and manoeuvring in accordance with the provisions of Appendix F that is part of Rule 1.1 of the Lakes A Zone (the full text version). Noise in accordance with the provisions of Appendix 5 of Volume 2 of the Lakes A Zone. Council shall reserve its control to the following matters: 1. Anticipated Environmental Results of this Schedule. 2. Design and external appearance of buildings and structures. 3. Landscape design and site layout. 4. Location and design of vehicular and pedestrian access to and from the site. 5. Parking and manoeuvring. 6. Management of noise. 7. Management of earthworks. 8. Natural hazards. Council may impose conditions in respect of the following matters: * The Anticipated Environmental Results of this Schedule. * Requiring buildings and structures to be of a similar or complementary design and appearance to existing buildings and structures on the same site. * Requiring buffers in the form of yards, screening and landscaping or a combination of these. * Specifying site layout of buildings, parking and vehicular circulation areas. * Requiring the activity to be located, where practicable, away from abutting or adjoining activities so as to protect the privacy of those activities. * Requiring additional parking where the activity is likely to generate a need for parking beyond the requirements of Appendix F that is part of Rule 1.1 of the Lakes A Zone (the full text version). * Specifying where on-site parking and manoeuvring areas are to be located and to require screening, fencing or landscaping to avoid adverse visual effects on neighbouring properties. * Noise controls and hours of operation. * Specifying soil retention or removal or landfilling methods. * Controlling the effects of activities in order to avoid or mitigate natural hazards. In addition to the above, Council may impose a bond to ensure satisfaction of conditions of consent and a charge to cover monitoring costs. ## A42.3 Restricted Discretionary Activities There are no Restricted Discretionary Activities. ## A42.4 Discretionary Activities A42.4.1 Any building or activity that is not a Permitted or Controlled Activity. ## A42.5 Non-Complying Activities There are no Non-Complying Activities. |  |
| 43.0 Tangata Whenua Structure Plans | ## 43.1 Permitted Activities No sites for Tangata Whenua Structure Plans have been identified within these Management Areas. |  |  |  |

Natural Hazards Chapter (NH) – Main part of the District Plan

| ###### Rule | Rules | Indicate whether the proposed work: - complies or will comply with the permitted rule, (demonstrate how in application) - requires land use consent. If so what type? - or alternatively that the rule is not applicable to the activity (N/A), For Council use - whether further information is required to determine whether permitted or otherwise |
| --- | --- | --- |
| Faults | NH-R1; Buildings and their use in the Fault Rupture Hazard Area or Possible Fault Rupture Hazard Area. Applicable Spatial Layers Fault Rupture Hazard Area, Possible Fault Rupture Hazard Area; 1.; Activity Status: Restricted Discretionary Where: - The building: - is a new building; or - has an increase in building importance level. - Except that this rule does not apply to: - a Building Importance Level 1 building; - a Building Importance Level 2 building in the Possible Fault Rupture Hazard Area; - a replacement building that does not increase the building importance level; or - buildings and their use on existing lots in Residential Zones, Commercial Zones, Business and Innovation Zones or the Lakes A Zone Settlement Management Area or Bush Settlement Management Area, where the recurrence interval of the active fault affecting the site is consistent with the minimum recurrence interval in the table below: Building Importance Category; Minimum Recurrence Interval – Existing Developed Areas. 2a and 2b; >2,000 years. 3; >5,000 years. 4; >20,000 years. Matters of Discretion: - The extent to which natural hazard risks are avoided or mitigated and whether the activity would worsen any natural hazard. - The extent to which the risk arising from locating a building within a Fault Rupture Hazard Area is managed, as set out in a natural hazard assessment report from a suitably qualified expert, identifying the potential location of the fault line, its recurrence interval and any subsequent building design and location requirements or restrictions on use.. |  |
| Flooding | NH-R4A; Buildings and their use in areas susceptible to flooding – Lower Flood Hazard Area. Applicable Spatial Layers All Zones; 1.; Activity Status: Permitted Where: - The building: - is located in a Lower Flood Hazard Area; and - is a new building; - has an increase in the building importance level; or - is an addition to an existing building. Performance Standards - Except as provided in b), the building has a floor level above the anticipated flood level (as relevant to the site) and allowing for freeboard that addresses imprecision in the flood level design estimate, construction tolerances and natural phenomena (such as waves, debris, aggradations, channel transition and bend effects) not explicitly included in flood level calculation. - The minimum floor level performance standard in a) shall not apply to: - Additions of less than 20m2 to an existing building where there is no increase in building importance level; - Buildings of building importance level 1 (building of low importance); or - Buildings within the Electricity Generation Core Site.; 2; Activity Status: Restricted Discretionary Where: Compliance is not achieved with the performance standards for rule NH-R4(1). Matters of Discretion: - Building in areas susceptible to flooding NH-MD1. NH-R4; Buildings and their use in areas susceptible to flooding – Higher Flood Hazard Area. Applicable Spatial Layers All Zones; 4.; Activity Status: Restricted Discretionary Where: - The activity is not permitted by rule NH-R4(1); and - Taking into account climate change based on RCP8.5 to the year 2130 (or the most recent national or regional guidance), the modelled 1%AEP flood depth from rivers, overland flow or lake inundation is greater than 300mm above ground level at the building site. - The building: - is located in a higher flood hazard area; and - is a new building; or - is an addition to an existing building of 20m2 or more; or - has an increase in the building importance level. Exception: This rule shall not apply to buildings within the Electricity Generation Core Site. Matters of Discretion - Building in areas susceptible to flooding NH-MD1. Refer to full text Advice Note: Any buildings and construction activities within Flood Protection and Drainage Applicable areas require Flood Protection and Drainage Bylaw authority from Bay of Plenty Regional Council to ensure flood protection and drainage infrastructure assets are not damaged or compromised. Regional Council’s interactive map shows which properties the Bylaws cover.. NH-R5; Changes to Overland Flowpaths. Applicable Spatial Layers Residential zones, Rural 2 Zone, City Centre Zones, Commercial Zones, Industrial Zones, Business and Innovation Zones, Lakes A Zone Settlement Management Area and Bush Settlement Management Area.; 1.; Activity Status: Restricted Discretionary Where: - A building or structure results in a change to the entry or exit point of an overland flowpath on a site, pipes or reduces the capacity of the overland flowpath; and - The activity is not granted a consent or a permit by the regional council that specifically authorises the modification of an overland flowpath or is for the maintenance, renewal or upgrade of Rotorua Lake Council’s urban stormwater network where the discharge is authorised by the regional council. Matters of Discretion - The extent to which natural hazard risks are avoided or mitigated and whether the activity would worsen any natural hazard.. |  |

I………………………………..(Applicant or Agent for Applicant) have completed the above checklist.  To the best of my knowledge have answered correctly and provided the information required with my application for a Project Information Memorandum or with my land use consent application.

SIGNED:	______________________________________________________	DATE: 	____________________________

lakes a zone - settlement management area - customer copy

All words shown in italics within the Lakes A part of the District Plan and the attached checklist are defined within the District Plan.  Those definitions most likely to need consulting while completing the attached checklist are reproduced below to assist with use of this checklist.  Please consult the full text within the District Plan for definitions not listed.

| AEP | : | Means Annual Exceedence Probability. A 2% AEP identifies a 2% (1 in 50) chance of there being an event (e.g. a flood) of this size or larger in any given year. Also known as a 50 year return period event. With regard to river AEP flood levels, the levels include margins to cover design assumptions, estimate imprecision, and artificial interferences with flow paths. With regard to lake levels, the levels include margins to cover estimate imprecision, waves and local wind set-up, seiche, and the length of time lake levels can remain high. The 2% AEP lake flood levels are: ###### Okareka – 355.20 metres RL Moturiki datum Tarawera – 299.40 metres RL Moturiki datum Okataina – 314.90 metres RL Moturiki datum Rotokakahi – 395.90 metres RL Moturiki datum Tikitapu – 419.50 metres RL Moturiki datum |
| --- | --- | --- |
| Building | : | Means any thing constructed, whether temporary or permanent, moveable or immovable, including but not limited to: - Additions to buildings; - Tanks, containers, reservoirs or swimming pools exceeding 25,000 litres capacity; - Cable cars, cable ways and similar devices; and - Any thing that is a structural part of, or structurally attached to, a building including equipment, devices, tanks, masts, aerials, poles and satellite dishes; But excludes - The interior of any building; - Scaffolding erected temporarily while in the course of construction or maintenance purposes; - Structures and Lakes Structures; - Connections between structures and buildings by way of cables, wires, ropes and the like; - Hard surfaces; - Platforms, bridges, deckings and the like where they are less than 1 metre from the ground; - Outdoor storage; - Septic tanks and infrastructure or utility services placed below ground level; and - Signs. |
| Building Platform | : | Means the area of land on which a building is built or is to be built and includes any on-site effluent disposal system (e.g. a septic tank and its disposal field). The platform does not have to be shaped but can follow the natural lie of the land. |
| Buffer | : | Means a separation distance or an area used to mitigate or avoid adverse effects that arise from one place being experienced in another. Special management may be required within the buffer, such as planting or exclusion of buildings. |
| Disturbance | : | Means the removal, clearance, destruction, cutting, crushing, desiccation or burning of exotic or indigenous vegetation; except: - For normal domestic-scale trimming and maintenance; and - The day to day maintenance of walking tracks; and - The collection of plant material for scientific purposes, and - The collection of plant material by the Tangata Whenua for maintaining traditional practices of rongoa (medicinal purposes), raranga (weaving) and mahi whakairo (carving). |
| Earthworks | : | Means the disturbance of land surfaces by excavation or filling, but excludes normal domestic and reserve gardening activities, normal turf and pasture maintenance and renovation practices, and the maintenance of walking tracks, farm and forestry tracks, driveways and roads. |
| Ground Level | : | Means the natural level of ground before any earthworks have taken place. It also means the level of the ground after any authorised earthworks have been carried out before any subdivision is completed pursuant to section 224(c) of the RMA. |

| Groundwater table | : | Means the surface of the watertable. For the purposes of subdivision and development: - Where the groundwater is influenced by a river or stream, either an identified winter groundwater level is used, or the groundwater table is established with reference to the water level in the river or stream; or - Where the groundwater is influenced by a lake, the groundwater table is established with reference to the maximum control level for a controlled lake, or the 2% AEP flood level for an uncontrolled lake. |
| --- | --- | --- |
| Hard Surface | : | Means an area of land that is paved with an all weather impermeable surface, such as concrete, asphalt or cobblestones, or covered with a decking or platform, but excluding: - Paved strips less than 1 metre in width; - Driveways less than 3.5 metres in width (for the purpose of Rule 11.0 but not for Rule 35.0); - Farm tracks less than 3.5 metres in width; - Decking or platforms less than 10 m2 in area; - Timber decking that has gaps between the boards, where the rainwater falls through the gaps to the ground; - Buildings; - Structures; - Lake Structures |
| Hard Surface Coverage | : | Means the portion of a site that may be paved or covered with hard surfaces. Any area provided for the purposes of access and including but not limited to roads, Maori roadways, rights-of-way, access lots, and access legs, shall be excluded from the calculation of hard surface coverage for any site. |
| Indigenous vegetation | : | Means any plant community containing indigenous species (which may include a canopy, subcanopy, understorey and ground cover as structural elements). It includes vegetation that has regenerated naturally or vegetation established with human assistance following disturbance or as mitigation for another activity. (Refer also to Indigenous Plant Species, Vegetation Unit and Vegetation Type.) |
| Lake Structures | : | Means any thing constructed, whether temporary or permanent, moveable or immovable, where any part intrudes into, or is placed over, the waters of a lake, or is on land that would be covered by water when the lake is at its 2% AEP flood level; and including: - Buildings - Structures - Walls and retaining walls built of any material including rock riprap or spawls But excluding - Boats, motor craft and vessels. |
| Site | : | Means: - An area of land which is the smaller land area of either: - Land comprised in a single allotment held in one certificate of title; or - Land comprised in a single allotment or the balance area on an approved subdivision scheme plan for which a separate certificate of title can be issued without further Council involvement. - An area of land comprising two or more adjoining allotments held in one certificate of title that cannot be treated separately without the Council granting its consent. - An area of land comprising two or more adjoining allotments held in two or more certificates of title where the titles are: - Subject to Section 37 of the Building Act 1991: “Construction of building on 2 or more allotments”; or - Held together in such a manner that they cannot be assessed individually without the consent of the Council. d) An area of Maori freehold land that either: - Is land created by way of partition under Sections 297 and 301 of Te Ture Whenua Maori Act 1993 (The Maori Land Act 1993) and held in one Maori Land Court Title, or - Is land defined by survey and created by way of partition into one parcel to be held by an owner who is a member of the same hapu, or owners who are members of the same hapu, or - Is land defined by survey and held in a Maori Land Court Title and for which ownership can be determined, or - Is land defined by survey and created by way of partition for a site for a dwelling under Section 296 of Te Ture Whenua Maori Act 1993 (The Maori Land Act 1993). |

|  |  | e) An area of Maori Customary Land. - Is land defined by survey and reserved under the Reserves Act 1977 or Te Ture Whenua Maori Act 1993 (The Maori Land Act 1993). Not withstanding paragraphs a-f above, where land has been subdivided under the cross lease or company lease systems (other than unit titles), a site means an area of land containing: - Building(s) for residential, commercial and industrial activities with any accessory building(s) and land exclusively set aside for the occupants/users of the building(s); or - Any share in the fee simple which creates a vacant part of the whole for future cross lease or company lease purposes; and Not withstanding paragraphs a-f above, any land subdivided under the Unit Titles Act 1972 (other than unit titles) a site shall mean either: - The area of land containing the principal unit (or proposed unit) on the unit plan, and any identified accessories to that principal unit, or in addition to the above, the following shall apply: ii) The underlying certificate of title of the land containing the unit titles, immediately prior to subdivision. In addition to the above, the following shall apply: - Where a site is crossed by a Zone boundary, the site is deemed to be divided into two or more sites by the Zone boundary; and ii) Where a site is divided by the District boundary, the area of the site remaining within the District shall be considered the site. |
| --- | --- | --- |
| Site Coverage | : | Means that portion of the site over which buildings may be constructed. Any area provided for the purposes of access and including but not limited to roads, Maori roadways, rights-of-way, access lots, and access legs, and any eaves of a building that do not exceed 0.6m in width shall be excluded from the calculation of site coverage. |
| Structures | : | Means any thing constructed, whether temporary or permanent, moveable or immovable, that is not a building and that is above ground level, including but not limited to: - Tanks, swimming pools or containers that exceed 5000 litres capacity; and - Walls or retaining walls that exceed 0.5 metres above ground level in height; - Fences with panels, or with boards, pickets or rails that are closer together than their widths; - Utility services; - Towers, pylons, poles or aerials; - Antennas, satellite dishes, rods; - Caravans, tents, or other structures intended to be habitable; - Constructions designed for children to play on; but excluding - Hard surfaces; - Lake structures; - Outdoor storage; - Road formations and access formations; Scaffolding or falsework erected temporarily while used in the course of construction or maintenance purposes. |
| Sunlight Control Plane | : | Means a surface through which no part of a building may protrude. It is defined by: - Drawing sunlight control lines commencing at a height of 2.5 metres above and along the affected site boundaries; and - Pointing the sunlight control lines into the site at a 28º angle above the horizontal at right angles along the affected site boundaries. The site boundaries affected by the sunlight control plane are those that are touched by the shaded portion of the Sunlight Control Plane Circle (i.e. between 135º and 225º from true north). Boundaries affected by the Sunlight Control Plane (refer to illustration examples below): - Identify true north on the site plan and orientate the Sunlight Control Plane Circle to the true north point. - Place the outside of the Sunlight Control Plane Circle to the inside of the site boundary. Move the sunlight control plane circle around the entire length of the site boundaries. The sunlight control plane shall apply only to the boundaries where the shaded area of the circle touches the site boundary. |

| Viewpoint | : | Means a viewpoint identified in the Viewpoint Map and the schedule of viewpoints attached thereto. Check with Planning Services Dept or Volume II full text Lakes A. The assessment of any effect on a view shall be confined to those effects within any pair of viewshaft bearings as identified in the Viewpoint Map and schedule, when seen from the viewpoint that forms the origin of that pair of viewshaft bearings. |
| --- | --- | --- |

Appendix K

See Lakes A zone - Volume II for details

Appendix F – As it applies to Lakes A zone for houses. For other activities see Appendix F of the District Plan

| Activity | Parking Performance Standard |
| --- | --- |
| Household units (including dwelling houses, subsidiary household units, residential accommodation and all like activities | 1 for each unit, except for the Lakes A zone where 2 parking spaces for each unit shall be provided. |